Ilia State University

Faculty of Business, Technology, and Education

School of Technology


P.P.A.M.S – Posture Pillow Attachment Monitoring System

Proposal

Team Members:
Jambuli Chkhikvadze
Samir Mansour
Giorgi Erkomaishvili

Computer Engineering Undergraduate Program

Senior Design Project A

Project Supervisors: Prof. Paata Gogishvili, Prof. George Modebadze

12/18/20205

Tbilisi, Georgia

Contents

Abstract	2

Introduction	3

3D device Diagram	4

Device Diagram 	4

Electrical Diagram	5

Custom Chapters	5

# Abstract

This project proposes the design and development of the Posture Pillow Attachment Monitoring System (P.P.A.M.S), a consumer wellness device intended to improve sitting posture awareness among office workers and university students who spend extended periods seated. Prolonged poor posture is a widespread problem that often develops unconsciously and may lead to long-term discomfort and reduced well-being. The primary objective of the project is to monitor seated posture in real time and provide timely feedback that encourages users to self-correct their posture before harmful habits becomes persistent.

The proposed system will consist of a removable chair attachment incorporating pressure sensors, embedded processing, and wireless communication capabilities. Sensor data will be processed by an embedded microcontroller and compared against a user calibrated baseline to identify sustained deviations from a correct sitting posture. When poor posture is detected, the system will provide gentle feedback through vibration and audio alerts. A companion mobile application will support system calibration, configuration, real-time monitoring, and visualization of posture history.

The project will employ standard embedded systems design practices, wireless communication technologies, and basic data-driven adaptation techniques. Expected outcomes include a functional hardware prototype, a mobile application, and complete technical documentation. The project has educational value by integrating hardware, software, and system design, and practical value by promoting healthier sitting behavior in everyday environments.

# Introduction

Prolonged sitting is a common part of modern academic and professional life, especially for university students and office workers who spend long hours at desks. Maintaining poor posture for extended periods can lead to discomfort, fatigue, and long-term musculoskeletal problems. A major challenge is that many individuals are unaware of their posture while working, allowing unhealthy habits to develop over time.

Existing solutions such as ergonomic chairs, lumbar supports, and wearable posture trackers attempt to address this issue, but each has limitations. Ergonomic furniture does not actively monitor user behavior, while wearable devices can be intrusive or uncomfortable for continuous use. These shortcomings indicate a need for a non-intrusive system that can monitor seated posture and provide timely feedback without disrupting daily activities.

The Posture Pillow Attachment Monitoring System is proposed as a removable chair attachment that integrates sensing, embedded processing, and wireless communication to improve posture awareness. The project fits within modern embedded systems and user-centered engineering practices, offering educational value through hands-on system integration and practical relevance by encouraging healthier sitting habits in everyday environments.

# 3D device Diagram

SCH shema 
Device Diagram

# Electrical Diagram

# Github Reposetory

This chapter provides detailed technical information and supporting materials for the project. All development resources are maintained in a dedicated GitHub repository to support collaboration, version control, and documentation.

The repository contains the project source code, including embedded firmware and application-related components, as well as the original drawing files referenced in the “Drawings” section. All diagrams and design images are organized in a separate folder dedicated to proposal and design assets.

Project documentation will also be included in the repository. This documentation will serve as a user and technical guide, describing system setup, calibration procedures, and maintenance requirements necessary for specialists to configure and support the final product. Additional materials such as selected snippets, charts, and appendixes may be included where they improve technical clarity.

The final project presentation and supplementary resources will be uploaded to the same repository upon project completion.

GitHub Repository:
https://github.com/BigMoon-103/PPAMS

# Industry Standards

The Posture Pillow Attachment Monitoring System (P.P.A.M.S) is designed with consideration of relevant industry standards and engineering best practices to ensure safety, interoperability, and technical reliability. Although the system is a consumer wellness device and not a medical product, recognized standards guide the design process and improve system quality.

Wireless communication follows IEEE 802.11 and IEEE 802.15 standards, which define Wi-Fi and Bluetooth technologies used by the ESP32 for device-to-mobile connectivity. Electrical safety and electromagnetic compatibility principles are informed by IEC 62368-1 and IEC 61000 standards, supporting safe low-voltage operation and noise-resistant design.

Printed circuit board layout and fabrication follow IPC-2221 guidelines to ensure proper trace spacing, component placement, and manufacturability. Embedded firmware development on the STM32 platform aligns with ARM CMSIS standards, providing a standardized interface to microcontroller peripherals and improving code portability.

Software structure and system requirements are guided by ISO/IEC/IEEE 29148 principles, while general networking behavior follows relevant RFC standards. These standards collectively ensure that the system is developed in line with modern embedded systems and IoT engineering practices.

# Gant Chart

# Budget

| Device | Number of Devices | Price Per Device (Gel) | Total Price (Gel) | Shop link | Comment |
| --- | --- | --- | --- | --- | --- |
| ESP32DevKitC | 1 | 22,50 ₾ | 22,50 ₾ | 1 |  |
| Mini Vibration Motors DC 3V-5V | 10 |  |  | 2 |  |
| Active Buzzer Module 3.3V-5V | 1 |  |  | 3 |  |
| STM32F4 Discovery / Dev Kit | 1 |  |  | 4 |  |
| 3000 mAh Rechargeable LiPo Battery | 1 |  |  | 5 |  |
| USB charging module and protection circuit | 1 |  |  | 6 |  |
| Cushion foam |  |  |  | 7 |  |
| Non-wettable protective fabric |  |  |  | 8 |  |
| Flex tape / reinforcement for sensor placement |  |  |  | 9 |  |
| Velcro straps or small clamps for chair fixation |  |  |  | 0 |  |
| Adhesive, fasteners, or small screws for mounting electronics |  |  |  | 11 |  |
| Resistors, jumper wires, and passive components |  |  |  | 12 |  |
| Wires and JST connectors |  |  |  | 13 |  |
| Force Sensitive Resistors |  |  |  | 14 |  |
| Flex Sensor 2.2" |  |  |  | 15 |  |
|  |  |  |  | 16 |  |
|  |  |  |  | 17 |  |

# References

[1] STM32F4 Discovery Board Datasheet, STMicroelectronics, [Online]. Available: https://www.st.com/resource/en/datasheet/stm32f407vg.pdf.

[2] ESP32-DevKitC V4 Datasheet, Espressif Systems, [Online]. Available: https://docs.espressif.com/projects/esp-dev-kits/en/latest/esp32/esp32-devkitc/user_guide.html

[3] Interlink Electronics, FSR™ Integration Guide, Document 94-00004 Rev B, 2023. [Online].Available: https://akizukidenshi.com/goodsaffix/94-00004_Rev_B%20FSR%20Integration%20Guide.pdf